ECSE 403 - Control

Laboratory 5 - Report

Mohamed Reda EL KHILI - 260678513

François-Eliott Rousseau - 260670000

Ismail Faruk - 260663521

- Considering the position as the output and the coefficients from the previous laboratory, we obtain the transfer function: G(s) = 1200s*(s+8.569).
We obtain the bode plot shown below with a gain margin of infinity and a phase margin of 19.8 deg.

Figure: Bode Diagram of the system

- We use Kp = 0.45 and we obtain a phase margin of 20.9 deg and a gain margin of infinity.

Figure: Open Loope Bode Diagram

  - Done

  - Done

  - The current phase margin of the open-loop system is 20.9 deg

  - Taking a safety margin of 5 degrees, the required phase is 44.1 deg.

  - We have a required phase of 44.1 degrees. Solving the equation:
ϕ = sin-1(α-1α+1). We get α = 5.579

  - By looking at the Bode plot we find a desired frequency wm of 35.2 rad/sec

  - We get, T = 1wm*α= 0.012.
Hence, our lead controller is Clead = 0.067s  + 10.012s + 1
With this lead compensator, we reach a phase margin of 57.8 deg.

Figure: Step response of proportional feedback controller, with and without the compensator - Model

Figure: Step response of the system to proportional feedback controller with Lead compensator - Physical

Figure: Step response of the system to proportional feedback controller without Lead compensator - Physical

- We can see that adding the lead compensator allows the system to get rid of the overshoot and to have a slower rise time by having a bigger Kp. We also have a bigger phase margin which makes the system more stable. We have a steady state error of 2.3%.
The bandwidth of the closed loop system is at 51 rad/s  (frequency at which the magnitude goes to -3dB).

  - Done

  - Taking the limit of s*H(s)*Klagas s goes to 0, we find:
Kv=Klag *63.02

  - We had a steady state error of 2.3% using the lead compensator. Hence, we now want a steady state error of 0.23% with the lead-lag compensator as asked. We thus get: ess = 0.23%
We thus obtain Klag=6.899

  - By looking at the bode plot, we find that at w = 6.92 rad/sec, we have the required phase margin of -110 deg.

  - We find using w = 6.92 rad/sec a dB-drop 17.8 dB and we get α=7.7625

  - We finally get T = 1.4451

Hence, our lead lag compensator is given by:
Clag= KlagTs + 1αTs + 1=6.898×1.4451s + 17.7625×1.4451×s + 1
Plotting the Bode Plot, we obtain a phase margin of 57.9.

Figure: step response of proportional feedback controller with LEAD and LEAD-LAG compensators

Figure: Ramp Response of proportional feedback controller with LEAD and LEAD-LAG compensators

Figure: Step response with Lead-Lag compensators

- The Lead-Lag has smaller rise but higher settling time than PID. Lead-Lag also has lower steady state error and no overshoot.

Figure: Lead-Lag compensators vs PID

Appendix

%% Q1
s = tf('s');
G = 1200/(s*(s+8.569));


%% Q2
H = 1200*0.45/(s*(s+8.569));
bode(H)
margin(H)

%% Q5
C = (0.067*s+1)/(0.012*s+1);

Z=C*H;
margin(Z);
bode(Z);

%% Q6
X = 6.899*(1.4451*s+1)/(1.4451*7.7625*s+1);
margin(Z*X)
bode(Z*X);

%% Q9
% time = (simout.time);
% H1 = (simout.signals.values); 
% H2 = (simout.signals.values); %kp = 0.178, ki = 0.092, kd = 0.0075

% used to find length
% length(H1); length 4722
% length(H2); length 4682
% length(time); length 4682
% H_t = length(H2); %4682
% 
% H3 = H1(1:H_t); %extract data of set size H_t
% H4 = H2(1:H_t); %extract data of set size H_t
% t = time(1:H_t);

plot(t,H3,t,H2,'r');
legend('Lead-Lag', 'PID');